System Architecture Design Document

Project Title: Autonomous Self-Healing Edge Security Gateway

Target Hardware: Raspberry Pi 5 (8GB RAM / Quad-Core Broadcom BCM2712 @ 2.4GHz)

System Class: Zero-Trust Multi-Agent Cyber-Physical Firewall

Executive Summary

Existing IoT edge defense frameworks rely on passive anomaly detection or static rule enforcement, creating single points of failure, cloud latency bottlenecks, and vulnerability to prompt injection attacks.

This document details the architectural design for an Autonomous Self-Healing Multi-Agent Security Gateway. Built upon a 3-node state-graph topology, this architecture separates local observation, cognitive planning, and sandboxed physical execution. By combining microsecond-level local ML detection with an adaptive edge-cloud hybrid agent loop, the gateway monitors system health, formulates mitigation policies, executes kernel-level network blocks, and validates post-execution metric recovery—100% autonomously, with full offline fallback capabilities.

1. High-Level System Architecture

The gateway operates on a closed-loop control model divided into three main operational layers:

- The Physical Sensing & Data Ingestion Layer: Monitors raw packet flows and converts metrics into normalized vector representations.

- The Local Anomaly Detection Layer: A lightweight Machine Learning model evaluates traffic to generate an anomaly score (st).

- The Multi-Agent Cognitive Orchestration Layer: A 3-node state graph (LangGraph) that manages environmental supervision, tactical planning, and sandboxed OS execution.

[ Incoming Network Data Packets ]

│

▼

[ Feature Extraction & Normalization ]

│

▼

[ LightGBM / TinyML Detector (INT8) ]

(Calculates Anomaly Score: s_t)

│

┌──────────────┴──────────────┐

│ s_t < 0.70                  │ s_t >= 0.70

▼                             ▼

[ System Normal ]           [ ALERT TRIGGERED ]

(Agents Asleep)                       │

▼

┌───────────────────────────────────────┐

│ 1. Network & Environment Supervisor  │

│    - Measures OS Health (CPU/RAM)     │

│    - Tests Cloud Connectivity         │

└───────────────────┬───────────────────┘

│

┌─────────────────────────┴─────────────────────────┐

│ Cloud Connected                                   │ Cloud Offline / DDoS

▼                                                   ▼

┌───────────────────────┐                           ┌───────────────────────┐

│  2. Planner Agent     │                           │  2. Planner Agent     │

│  (Cloud LLM API:      │                           │  (Local SLM:          │

│   GPT-4o-mini/Haiku)  │                           │   Phi-3-Mini INT4)    │

└───────────┬───────────┘                           └───────────┬───────────┘

│                                                   │

└─────────────────────────┬─────────────────────────┘

│ Passes Strict JSON Schema

▼

┌───────────────────────────────────────┐

│ 3. Execution Guard Agent              │

│    - Runs inside gVisor Sandbox       │

│    - Fires eBPF / iptables Tools      │

└───────────────────┬───────────────────┘

│

▼

┌───────────────────────────────────────┐

│ Post-Execution Metric Validation      │

│ (Supervisor confirms OS recovery)     │

└───────────────────────────────────────┘

2. Mathematical Foundation: The Anomaly Ignition Model

The multi-agent system remains dormant during normal network operations to preserve hardware resources on the Raspberry Pi 5. The workflow is triggered strictly when an anomaly score st breaches a designated safety threshold τalert.

Step 1: Feature Vector Normalization

Incoming raw packet properties xt∈Rn (e.g., packet rate, flow duration, TCP flag entropy) are normalized against a benign baseline dataset:

xt=Nxt=xt-μσ

Where μ and σ represent the baseline mean and standard deviation vectors.

Step 2: Anomaly Score Calculation

The normalized vector xt is evaluated by a quantized local model (LightGBM INT8) to compute the bounded probability st∈0,1 that the current traffic flow is non-benign:

st=1-Pyt=benign∣xt,θ=maxy∈Y∖benignPyt=y∣xt,θ

Step 3: Threshold Trigger Rule

System State=Dormant (Log Benign),if st<τalert default τ=0.70Invoke Multi-Agent Swarm,if st≥τalert

3. Detailed Component Breakdown

Component 1: The Local Anomaly Detector

- Technology: LightGBM (Quantized INT8) / TinyML Feedforward Neural Network.

- Footprint: <100 KB RAM footprint; sub-millisecond inference time.

- Function: Evaluates network packet streams continuously. By avoiding heavy deep learning models (like LSTMs or CNNs), it keeps background CPU usage near 0%, preserving resources for network traffic handling.

Component 2: The Multi-Agent State Graph (LangGraph Engine)

The orchestration framework is built using a 3-node directed state graph, where each node has a distinct, isolated responsibility:

┌───────────────────────────────────────┐

│ 1. Network & Environment Supervisor  │

│    (Reads System Stats / Routes Logic)│

└───────────────────┬───────────────────┘

│

▼

┌───────────────────────────────────────┐

│ 2. Threat Profiler / Planner Agent    │

│    (Determines Strategy via SLM/LLM)  │

└───────────────────┬───────────────────┘

│

▼

┌───────────────────────────────────────┐

│ 3. Execution Guard Agent              │

│    (Enforces Firewall via Sandbox)    │

└───────────────────────────────────────┘

Node 1: Network & Environment Supervisor Agent

- Role: Observation, Gatekeeping, and Verification.

- Execution: Native Python script (No AI inference cost).

- Core Responsibilities:

  - Connectivity Check: Continuously tests the cloud API endpoint. If latency spikes or the connection fails (e.g., due to a DDoS attack), it instructs the Planner to switch to Offline Mode.

  - Data Serialization: Compresses st, system CPU/RAM usage, and network flow properties into a compact JSON state object.

  - Verification Loop: Re-evaluates system health metrics after the Execution Guard Agent applies a firewall rule to confirm whether CPU load and network drop rates have returned to normal.

Node 2: Threat Profiler / Planner Agent

- Role: Cognitive Analysis and Policy Formulation.

- Execution:

  - Online Mode: Calls a lightweight cloud LLM API (GPT-4o-mini or Claude 3.5 Haiku).

  - Offline Mode: Uses a local Microsoft Phi-3-Mini (3.8B INT4) model running via Ollama.

- Core Responsibilities:

  - Evaluates the anomaly payload to identify the specific attack vector (e.g., SYN Flood, SSH Brute Force, Port Scan).

  - Generates a structured mitigation strategy in strict JSON schema format, detailing the target IP, port, and action (e.g., {"action": "block_ip", "target": "192.168.1.105", "duration": "3600s"}).

Node 3: Execution Guard Agent

- Role: System Tool Enforcement and Sandboxed Actuation.

- Execution: Isolated inside a zero-trust container sandbox (gVisor).

- Core Responsibilities:

  - Receives the validated JSON schema from the Planner Agent.

  - Maps parameters to system commands using local Python tools.

  - Executes low-level Linux kernel modifications using iptables or eBPF (Extended Berkeley Packet Filter) hooks.

  - Adversarial Safety: Operates completely isolated from raw network input data, preventing Prompt Injection attacks from reaching system execution layers.

4. Operational Workflows & Edge Cases

Workflow A: Normal Attack Mitigation Loop (Online)

- Sensing: LightGBM detects an SSH brute-force attempt (st=0.88).

- Supervisor Routing: Captures st, verifies the cloud API is reachable, and constructs a state payload.

- Planning: The cloud LLM receives the payload and returns a JSON mitigation strategy: {"action": "drop_ip", "ip": "10.0.0.45"}.

- Execution: The Execution Guard Agent runs sudo iptables -A INPUT -s 10.0.0.45 -j DROP inside the gVisor sandbox.

- Validation: Control loops back to the Supervisor Agent. It confirms system stats have normalized and marks the state as RESOLVED.

[Packet Stream] -> [LightGBM (s_t=0.88)] -> [Supervisor (Online)]

│

▼

[Resolved State] <- [Supervisor Check] <- [Execution Guard] <- [Cloud LLM API]

Workflow B: Cloud Outage / DDoS Mitigation Loop (Offline Fallback)

- Sensing: A volumetric SYN flood attacks the network (st=0.95).

- Supervisor Routing: The Supervisor pings the cloud endpoint, hits a timeout, and flags an Offline State.

- Fallback Planning: The Supervisor wakes up the local Phi-3-Mini (INT4) instance running via Ollama in the Raspberry Pi's RAM.

- Local Reasoning: Phi-3-Mini processes the JSON payload locally and outputs a kernel drop instruction.

- Execution & Validation: The Execution Guard applies the local firewall block, and the Supervisor verifies system recovery entirely offline.

[DDoS Attack] -> [LightGBM (s_t=0.95)] -> [Supervisor (Timeout Detected)]

│

▼

[Resolved State] <- [Supervisor Check] <- [Execution Guard] <- [Local Phi-3-Mini (Ollama)]

Workflow C: Power Failure & Energy Management Protocol

To ensure resilience during hardware power cuts, the gateway incorporates a 10-Point Emergency Power Protocol:

[Grid Power Failure] ──► [UPS Battery HAT Activates]

│

▼

[Supervisor Reads I2C Battery Level]

│

┌──────────────────┴──────────────────┐

│ Battery > 20%                       │ Battery <= 20%

▼                                     ▼

[Dim Non-Essential Interfaces]        [Flush System Logs to NVMe]

[Restrict AI to Critical Alerts]       [Trigger Graceful Kernel Shutdown]

- Uninterruptible Power Supply (UPS): The Raspberry Pi 5 runs on a dedicated UPS Battery HAT.

- Low-Power Mesh Network: Smart IoT nodes communicate via battery-efficient Zigbee or LoRaWAN protocols.

- Node Battery Backup: Field sensors use internal lithium/coin-cell backups.

- Store-and-Forward Buffering: If Wi-Fi drops, sensors temporarily buffer traffic logs in onboard flash memory.

- Log Re-Synchronization: Upon reconnection, buffered logs are transmitted to the gateway without data loss.

- I2C Power Monitoring: The Supervisor Agent reads battery charge states over the I2C bus.

- Resource Scaling: On battery power, non-essential interfaces and LED indicators are disabled.

- Priority Inference: AI reasoning is restricted to high-priority threats (st≥0.85) to conserve energy.

- Volatile Storage Flush: If battery levels drop below 20%, system logs and state tables are written to non-volatile storage.

- Graceful Kernel Shutdown: The Supervisor initiates a safe Linux shutdown (sudo shutdown -h now) to prevent SD card or NVMe filesystem corruption.

5. Architectural Comparison Matrix

| Feature / Metric | Base Paper Approach | Recommended Multi-Agent Architecture |
| --- | --- | --- |
| System Topology | Monolithic Single-Track Flow | 3-Node Graph (Supervisor-Planner-Executor) |
| Local Anomaly Detector | CNN + LSTM / Standalone LSTM [source: 3] | LightGBM (INT8) (Quantized Ensemble) |
| Detector Footprint | >1,300 MB RAM; High CPU Spikes [source: 368-369] | <100 KB RAM; Microsecond Inference |
| Cloud Dependency | Fixed Cloud API Dependency [source: 368-369] | Dynamic Edge-Cloud Hybrid (Offline Local SLM) |
| Execution Safety | Direct Script Parsing [source: 3478, 4614] | Sandboxed gVisor Container + Pydantic Schema Validation |
| Post-Execution Check | None (Blind Actuation) | Closed-Loop Metric Recovery Validation |
| Prompt Injection Risk | High (Unfiltered Data Pipelines) | Zero (Isolates Execution Agent from Raw Data Input) |

6. Implementation Hardware Specifications

- Processing Unit: Broadcom BCM2712 Quad-Core ARM Cortex-A76 @ 2.4GHz

- Memory: 8GB LPDDR4X SDRAM

- Operating System: 64-bit Raspberry Pi OS (Debian Bookworm)

- Local Inference Engine: Ollama / llama.cpp

- Agent Framework: LangGraph / Pydantic

- Container Isolation: gVisor (runsc runtime)

- Power Hardware: 27W USB-C PD Power Supply with 5V/5A Uninterruptible Power Supply (UPS) HAT